# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply and Delivery of 30 Night Vision Thermal Binoculars

Publication reference: EUBAM-26-16

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | NIGHT VISION THERMAL BINOCULARS 1/OPTICAL SYSTEM: - Detector Resolution: 384 × 288 minimum - Lens (Focal Length): minimum 25 mm - Field of View (FOV): 7.5° × 5.7° minimum - Human Target Detection Distance: 1 800 m minimum to 2,500 m maximum 2/THERMAL DETECTOR: - Detector Type: Vanadium Oxide (VOx) or equivalent - Cooling: Uncooled Focal Plane Array or equivalent system - Spectral Response: 8–14 μm - NETD: < 35 mK or < 15 mK - Pixel Pitch: 12 μm 3/IMAGE DISPLAY: - Display - Resolution: 1920 × 1080 minimum - Type: integrated TFT or OLED or AMOLED or equivalent - Size: from 0.3 to 0.6 -inch diagonal - Digital Zoom: 1× / 2× / 4× / 8× - Brightness (Luminance): Either manual 1–5 adjustable levels minimum, or automatic image optimization function that continuously adjusts the thermal image without requiring the user to intervene manually. - Contrast: Either manual 1–5 adjustable levels minimum, or automatic image optimization function that continuously adjusts the thermal image without requiring the user to intervene manually. 4/VIDEO & IMAGE FUNCTIONS: - photo capture and video recording - local playback of recorded videos and images - Storage support for videos and photos 5/CONNECTIVITY & FUNCTIONS: - Wi-Fi - Snapshot Capture - Automatic Power-Off 6/MEMORY: - Internal Storage: Minimum 64 GB 7/POWER SUPPLY: - Battery Capacity: 4,000 mAh to 5 000 mAh - Battey type: rechargeable Li-ion - Operating Time: 4 hours continuous operation minimum - USB charger 8/PHYSICAL CHARACTERISTICS: - Net Weight: from 0.40 to 0.99 kg 9/DOCUMENTATION: - User manual provided with each delivered night vision goggle. - Language: English: mandatory; Arabic: optional 10/WARRANTY : - Minimum factory warranty of 1 year All equipment shall be commercially available, non-lethal and intended for civilian operational use in border monitoring and inspection activities. Equivalent or superior specifications meeting the functional requirements shall be accepted. INCOTERM: DDP EUBAM LIBYA PALM CITY/TRIPOLI LIBYA QTY: 30 (THIRTY) |  |  |  |