# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Maintenance of EUBAM Libya Mission’s Vehicles	p 1 /23

Publication reference: EUBAM-26-04

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
|  | Maintenance Category A, including spare parts/consumables for Toyota Land Cruiser B6 Trojan, TLC 200, Armoured, Diesel, Automatic, 2018 As per Appendix 1, Section 1 |  |  |  |
|  | Maintenance Category C, including spare parts/consumables for Toyota Land Cruiser B6 Trojan, TLC 200, Armoured, Diesel, Automatic, 2018 As per Appendix 1, Section 1 |  |  |  |
|  | Maintenance Category A, including spare parts/consumables for Toyota Land Cruiser B6 Trojan, TLC 200, Armoured, Petrol, Automatic, 2022 As per Appendix 1, Section 1 |  |  |  |
|  | Maintenance Category C, including spare parts/consumables for Toyota Land Cruiser B6 Trojan, TLC 200, Armoured, Petrol, Automatic, 2022 As per Appendix 1, Section 1 |  |  |  |
|  | Maintenance Category E, including spare parts/consumables for Toyota Urban Cruiser, Soft skin, Petrol, Automatic, 2023 As per Appendix 1, Section 2 |  |  |  |
|  | Maintenance Category G, including spare parts/consumables for Toyota Urban Cruiser, Soft skin, Petrol, Automatic, 2023 As per Appendix 1, Section 2 |  |  |  |
|  | Maintenance Category E, including spare parts/consumables for Renault Duster, Soft skin, Petrol, Manual, 2022 As per Appendix 1, Section 2 |  |  |  |
|  | Maintenance Category G, including spare parts/consumables for Renault Duster, Soft skin, Petrol, Manual, 2022 As per Appendix 1, Section 2 |  |  |  |
|  | Maintenance Category E, including spare parts/consumables for Dodge RAM 1500, Soft skin, Petrol, Automatic, 2012 As per Appendix 1, Section 2 |  |  |  |
|  | Specialized Manpower For Unforeseen Maintenance (Not Part Of The Scheduled Maintenance) |  |  |  |
| Spare Parts for Toyota Land Cruiser B6 Trojan, TLC 200, Armoured (Diesel & Petrol) For Unforeseen Maintenance (Not Part Of The Scheduled Maintenance) |  |  |  |  |
|  | Pad kit, disc brake, front |  |  |  |
|  | Disc, front |  |  |  |
|  | Cylinder assy.disc brake (LH) front |  |  |  |
|  | Cylinder assy.disc brake (RH) front |  |  |  |
|  | Pad kit, disc brake, rear |  |  |  |
|  | Disc, rear |  |  |  |
|  | Cylinder assy.disc brake (LH) rear |  |  |  |
|  | Cylinder assy.disc brake (RH)rear |  |  |  |
|  | Shoe assy, parking brake Part number: 46530-34010 |  |  |  |
|  | Absorber assy, shock, front (LH) |  |  |  |
|  | Absorber assy, shock, front (LR) |  |  |  |
|  | Absorber assy, shock, rear (LH) |  |  |  |
|  | Absorber assy, shock, rear (LR) |  |  |  |
|  | V - belt Part number: 99369-k2250 |  |  |  |
|  | Radiator assy 16400-51051 |  |  |  |
|  | Fan 16361-50110 |  |  |  |
|  | Thermostat 90916-03089 |  |  |  |
|  | Tensioner assy, v-ribbed belt 16620-51011 |  |  |  |
|  | Pulley sub-assy, idler, no2 16603-51010 |  |  |  |
|  | Alternator assy 27060-51010 |  |  |  |
|  | Starter assy 28100-51072 |  |  |  |
|  | Pump assy injection or supply 22100-51032 |  |  |  |
|  | Cooler assy, fuel 23920-06028 |  |  |  |
|  | Injector assy 23670-59037 |  |  |  |
|  | Bearing front & RH or LH 43570-60031 |  |  |  |
|  | Hub & bearing rear axle RH 42450-60070 |  |  |  |
|  | Hub & bearing rear axle LH 42460-60030 |  |  |  |
|  | Shaft rear axle shaft LH or RH 42301-60030 |  |  |  |
|  | Shaft assy, propeller, front 37140-60590 |  |  |  |
|  | Carrier assy, differential rear 41110-60c01 |  |  |  |
|  | Arm assy front suspension upper, RH 48610-60060 |  |  |  |
|  | Arm assy front suspension upper, LH 48630-60030 |  |  |  |
|  | Arm sub-assy, front RH 48068-60030 |  |  |  |
|  | Arm sub-assy, front LH 48069-60030 |  |  |  |
|  | Tube assy, fuel tank vent 77010-60160 |  |  |  |
|  | Compressor assy cooler 88320-6a320 |  |  |  |
|  | Condenser assy, cooler 88460-60400 |  |  |  |
|  | Blower assy,w/shrouo 88590-60082 |  |  |  |
|  | LINK ASSY, POWER STEERING (Diesel and Petrol) 44200-60140 |  |  |  |
|  | Pump assy, vane 44310-60620 |  |  |  |
|  | Arm assy, lower control, (LH) 48720-60070 |  |  |  |
|  | Arm assy, lower control, (LR) 48720-60070 |  |  |  |
|  | Arm assy, upper control, rear (LH) 48710-60131 |  |  |  |
|  | Arm assy, upper control, rear (RH) 48710-60131 |  |  |  |
|  | Rod Assy, rear lateral control 48740-60150 |  |  |  |
|  | Bush, stabilizer (for rear) 48815-60221 |  |  |  |
|  | Shaft Assy, front driver (RH) 43430-60071 |  |  |  |
|  | Shaft Assy, front driver (LH) 43430-60071 |  |  |  |
|  | Shaft Assy, propeller, front 37140-60600 |  |  |  |
|  | Shaft Assy, propeller, rear 37110-60b80 |  |  |  |
|  | Strainer Assy, valve body oil 35330-60070 |  |  |  |
|  | Tube Assy, fuel tank vent 77010-60181 |  |  |  |
|  | Element Assy, fuel filter 23390-51070 |  |  |  |
|  | Oil filter 04152-38020 |  |  |  |
|  | Air filter 17801-51020 |  |  |  |
|  | Filter, clean air 87139-76010 |  |  |  |
|  | Turbocharger suassy 17201-51021 |  |  |  |
|  | Turbocharger suassy,no2 17208-51011 |  |  |  |
|  | Valve, cooler expansion 88515-50190 |  |  |  |
|  | Plug, spark 90919-01191 |  |  |  |
|  | Coil, spark 90919-02256 |  |  |  |
|  | Injector Assy, fuel (Petrol) 23209-39165 |  |  |  |
|  | Tube assy, fuel suction w/pump &gage (Petrol) 77020-60550 |  |  |  |
|  | Canister Assy, charcoal 77740-60550 |  |  |  |
|  | Pump Assy, vane (Petrol) 44310-60490 |  |  |  |
|  | Battery 12v-70 |  |  |  |
| Spare Parts for Renault Duster, Soft skin, Petrol, Manual, 2022 For Unforeseen Maintenance (Not Part Of The Scheduled Maintenance) |  |  |  |  |
|  | Oil filter 8200768927 |  |  |  |
|  | Air filter 165467674r |  |  |  |
|  | Fuel filter 164039594r |  |  |  |
|  | A/c filter interior 8201153808 |  |  |  |
|  | Fuel filter 164000797r |  |  |  |
|  | Rear brake pads (shoe) 440607493r |  |  |  |
|  | Front stabilizer bush 546114237r |  |  |  |
|  | Front brake pads(set) 440603905r |  |  |  |
|  | Fr/rr stabilizer link 551103022r (8200814411 / 546180001r) |  |  |  |
|  | Rear shock absorber 8200811407 / 562103964r) |  |  |  |
|  | Front shock absorber 543026656r |  |  |  |
|  | Wiper blade fr lh 288901088r |  |  |  |
|  | Wiper blade fr rh 288900610r |  |  |  |
|  | Clutch kit (cover, disc) 302051060r |  |  |  |
|  | Clutch bearing 306202899r |  |  |  |
|  | Battery 12v-70 ah 720 aen |  |  |  |
| Spare Parts for Toyota Urban Cruiser For Unforeseen Maintenance (Not Part Of The Scheduled Maintenance) |  |  |  |  |
|  | Oil filter |  |  |  |
|  | Air filter |  |  |  |
|  | Fuel filter |  |  |  |
|  | A/c filter interior |  |  |  |
|  | Fuel filter |  |  |  |
|  | Rear brake pads(shoe) |  |  |  |
|  | Front stabilizer bush |  |  |  |
|  | Front brake pads(set) |  |  |  |
|  | Fr/rr stabilizer link |  |  |  |
|  | Rear shock absorber |  |  |  |
|  | Front shock absorber |  |  |  |
|  | Wiper blade fr lh |  |  |  |
|  | Wiper blade fr rh |  |  |  |
|  | Battery 12v-45 |  |  |  |
|  | Tire |  |  |  |
| Spare Parts for Dodge RAM 1500, Soft skin, Petrol, Automatic, 2012 For Unforeseen Maintenance (Not Part Of The Scheduled Maintenance) |  |  |  |  |
|  | Link set front |  |  |  |
|  | Tensioner |  |  |  |
|  | Break discs set front |  |  |  |
|  | Break discs set rear |  |  |  |
|  | Hub and bearings |  |  |  |
|  | Wheel discs and tires front |  |  |  |
|  | Wheel discs and tires rear |  |  |  |

Other General Requirements:

- Communication with the contractor: documents in writing in English; oral communication in Arabic and English.

- The Contractor shall, if required when carrying over repair and maintenance services, be able to recover and transport any vehicle to the workshop in Libya.

- Schedule: The Contractor shall carry over maintenance services by executing the scheduled maintenance tasks considering the conditions of the vehicles and their mileage – including the substitution of parts with genuine spares. In doing so, the Contractor shall ensure that maximum one vehicle at the time is under scheduled, or preventive maintenance, so that the remaining vehicles are available for use if not under (unforeseeable) repair. To such extent, within 15 (fifteen) days from the start of Contract implementation, the Contractor shall propose to the Contracting Authority a calendar of activities detailing when the vehicles are scheduled for maintenance, including the pick-up and drop-off times.

- Stocks: The Contractor shall maintain stocks of genuine spare parts (i.e., original spare parts or parts that are recommended by the relevant manufacturer) and quality consumables that will enable the timely and quality performance of the maintenance services.

- Consumables: All the consumables needed to perform the maintenance services, including but not limited to filters, special liquids, etc. will be considered as included in the fee.

- Additional repairs/works that cannot be foreseen at the time of this Contract shall be managed on a case-by-cases basis.

- Transport: The Contractor is the sole responsible for the transport of vehicles from/to the Contracting Authority’s location(s) to/from the Contractor’s facilities.

- Inspection of the works and of the Contracting Authority’s vehicles: The Contractor is the sole responsible for performing the works. However, the Contracting Authority reserves the right to access the Contractor’s facilities and inspect both the works (including the spares/consumables to be used by the Contractor for the works) and the vehicles under maintenance/repair.

- End of works: The Contractor shall notify the Contracting Authority about the end of works as soon as the vehicle is maintained/repaired, fit for use and ready to be returned. Delays, i.e. late return of vehicles with respect to the agreed schedule as specified in the Contracting Authority’s order, may be subject to penalties.

- Acceptance of services: The Contractor shall perform all services to the highest standard of quality pertaining to the automotive industry, based on the vehicle road worthiness testing to international acceptable standards. After each work, the Contractor shall conduct adequate tests – e.g., road test with check of brakes, suspension, steering, clutch/manual transmission or automatic transmission, as per the relevant Manufacturer’s recommendation and/or work order – to ensure that the vehicle is returned fully operational and efficient. Tests shall be reported.

- Warranty: All maintenance and repair services performed shall be warranted by the Contractor in terms of both labour and spares/consumables. The warranty period shall never be less than 5,000 (five thousand) Km or 6 (six) months, whichever comes first, following the acceptance of the works by the Contracting Authority. Within 15 (fifteen) days from the start of Contract implementation, the Contractor shall propose to the Contracting Authority a form to be used in case of warranty claim. The Contracting Authority reserves the right to request the revision of the form in case it is deemed not complete and/or satisfactory for the objective of the Contract.

- Origin and quality of spares/consumables: The Contractor shall use only genuine, original/approved spares and consumables. In case genuine original approved parts are not available (e.g., because they are out of production), the Contractor shall propose to the Contracting Authority the use of equivalent, high-quality spares/consumables. The origin and quality of any spare parts/consumables used to perform the works must be attested by appropriate evidence. The manufacturer and part number(s) (P/N) of the concerned spares/consumables shall be indicated in the relevant report.

- The Contractor shall be the sole responsible for the vehicles from the moment they are handed over to the Contractor from the Contracting Authority to the moment they are returned after the works acceptance, as well as during the works when conducted at the Contracting Authority’s facilities. When the vehicle is under the Contractor’s responsibility, any damage or loss that might occur to the vehicle’s internal and/or external parts shall be repaired/rectified by the Contractor at their own cost. Also, during the work, the interiors of the vehicles will be always protected using seat covers and other protections to ensure that the interior furnishings are not damaged or soiled in any way.

- The Contractor shall not touch or tamper with any part of a vehicle that does not require inspection as per the objective of the Contract. The Contractor shall be responsible for the repair or replacement of any parts that are damaged through such unauthorized touching or tampering.

- In case of works conducted at the Contracting Authority’s facilities, the Contractor shall bear all the risks related to the goods needed for the works until provisional acceptance at destination. Such goods shall be adequately packaged to prevent their damage or deterioration during the transit to their destination.

- Management structure: All Contractual documentation, including quotations, invoices, reports, etc. will be checked for the conformity, assessed, proposed for amendment and approved by the Contracting Authority’s Logistics officer or designated alternative.

- Staff: The Contractor shall ensure that the manpower providing services have the relevant experience and are qualified to a level of acceptable to the vehicle manufacturer's satisfaction to be able to perform the services in-a professional manner.

- The Contractor shall also appoint a Point of Contact (PoC) who shall act as the only interface with the Contracting Authority. The PoC shall be fluent in English and shall have the experience and the education to effectively and efficiently manage all Contractor’s obligations foreseen by the Contract to achieve satisfactory results.

- In relation to the vehicle’s recovery, safe and guarded parking space for the vehicles shall be granted by the Contractor from the moment the vehicles are admitted into the workshops as soon as possible after the pick-up, until their handover back to the Contracting Authority. All building facilities (including, but not limited to, workshops, parking lots, spare parts stores) shall have limited and guarded access; furthermore, while under maintenance/repair, vehicles will not be allowed to be taken outside the Contractor’s controlled premises, for whatever reason, without the Contracting Authority’s prior authorization.

- The contractor must be able to have mobile workshops in case of emergency situations.

- The contractor shall be able provide intermediate maintenance, preventive maintenance and scheduled inspections/tests; Repair/replace unserviceable parts, assemblies, subassemblies and components; refinish, fabricate parts, and make modifications; repair accessories and auxiliary equipment, and body structural repair.

APPENDIX 1

Fleet Regular Maintenance Schedule

| Section 1: Armoured Vehicles Maintenance Schedule |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I = Inspect, C = Cleaning, D = Drain, R = Replace, T = Tighten, L = Lubricate |  |  |  |  |  |  |  |  |  |  |  |
| Maintenance interval x 1000 km | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 55 | 60 |
| Maintenance Category | A | B | A | A | C | A | A | B | A | A | D |
| Engine Compartment |  |  |  |  |  |  |  |  |  |  |  |
| Engine oil incl. Filter and plug gasket | R | R | R | R | R | R | R | R | R | R | R |
| V-Belt condition | I | I | I | I | I | I | I | T | I | I | I |
| Intercooler and air hoses | I | I | I | I | I | I | I | I | I | I | I |
| Cooling and heater system | I | I | I | I | I | I | I | 1 | I | I | I |
| Radiator and condenser incl. Connections | I | I | I | I | I | I | I | I | I | I | I |
| Coolant | I | I | I | I | I | I | I | I | I | I | R |
| Function of battery management | I | I | I | I | I | I | I | I | I | I | l |
| Batteries and connections | I | I | I | I | I | I | I | I | I | I | I |
| Spark plugs | n/a | n/a | n/a | n/a | R | n/a | n/a | n/a | n/a | n/a | R |
| Fuel filter | n/a | R | n/a | n/a | R | n/a | n/a | R | n/a | n/a | R |
| DIESEL drain water | D | n/a | D | D | n/a | D | D | n/a | D | D | n/a |
| Fuel hoses, tubes and clamps | I | I | I | I | I | I | I | I | I | I | I |
| Cyclone filter/ snorkel | I | I | I | I | I | I | I | I | I | I | I |
| Air filter cleaning, inspect / replace | C | R | C | C | R | C | C | R | C | C | R |
| Fire extinguisher incl. Seal of switch | I | I | I | I | I | I | I | I | I | I | I |
| Brake fluid | I | R | I | I | R | I | I | R | I | I | R |
| Power steering/ power steering oil | I | I | I | I | I | I | I | I | l | I | I |
| Engine operation / timing chain (sound) | I | I | I | I | I | I | I | I | I | I | I |
| Engine mountings | I | I | I | I | I | I | I | I | I | I | I |
| Visual check of engine compartment | I | I | I | I | 1 | I | I | I | I | I | I |
| Body |  |  |  |  |  |  |  |  |  |  |  |
| Ballistic glass | I | I | I | I | I | I | I | I | I | I | I |
| Glass sealings | I | I | I | I | I | I | I | I | I | I | I |
| Tailgate lock/ bonnet (hood) lock | I | I | I | I | I | I | I | I | I | I | I |
| Emergency exit lock tailgate | I | I | I | I | I | I | I | I | I | I | I |
| Doors and entries for damages | I | I | I | I | I | I | I | I | I | I | I |
| Door locks, central locking | I | I | I | I | I | I | I | I | I | I | I |
| Inner and outer mirrors | I | I | I | I | I | I | I | I | I | I | I |
| Adjustment of all doors/ readjust | I | I | I | I | I | I | I | I | I | I | I |
| Door hinges/ inspects and tighten | T | T | T | T | T | T | T | T | T | T | T |
| Doors and entries sealings | I | I | I | I | I | I | I | I | I | I | I |
| Window lifter/ hydraulic tailgate | I | I | I | I | I | I | I | I | I | I | I |
| Interbody |  |  |  |  |  |  |  |  |  |  |  |
| Brake drums | I | I | I | I | I | I | I | I | I | I | I |
| Brake disc/rotors, pads front | I | I | I | I | I | I | I | I | I | I | I |
| Brake disc/rotors, pads rear | I | I | I | I | I | I | I | I | I | I | I |
| Brake hoses and pipes | I | I | I | I | I | I | I | I | I | I | I |
| Diesel vacuum pump | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| Steering wheel/ steering free play | I | I | I | I | I | I | I | I | I | I | I |
| Steering stops | L | L | L | L | L | L | L | L | L | L | L |
| Driving shaft boots | I | I | I | I | I | I | I | I | I | I | I |
| Suspension ball joints | I | I | I | I | I | I | I | I | I | I | I |
| Anti roll bars and bushes front and rear | I | I | I | I | I | I | I | I | I | I | I |
| Shock absorber front and rear | I | I | I | I | I | I | I | I | I | I | I |
| Coil (leave) springs front and rear | I | I | I | I | I | I | I | I | I | I | I |
| Trailing and control! Arms front axle | I | I | I | I | I | I | I | I | I | I | I |
| Training links, rear axle mounts rear | I | I | I | I | I | I | I | I | I | I | I |
| Wheel nuts | T | T | T | T | T | T | T | T | T | T | T |
| Rim nuts (160Nm) | T | T | T | T | T | T | T | T | T | T | T |
| Tire pressure and tire condition | I | I | I | I | I | I | I | I | I | I | I |
| Change wheels- front to rear | n/a | I | n/a | n/a | I | n/a | n/a | I | n/a | n/a | I |
| Balance wheels | I | I | I | I | I | I | I | I | I | I | I |
| Fuel tank/ fuel filler flap | I | I | I | I | I | I | I | I | I | I | I |
| Bulbar, winch, tow-hocks | I | T | I | I | T | I | I | T | I | I | T |
| Park brake linings | n/a | I | n/a | n/a | I | n/a | n/a | I | n/a | n/a | I |
| Steering gear | I | I | I | I | I | I | I | I | I | I | I |
| Propeller shaft grease | L | L | L | L | L | L | L | L | L | L | L |
| Propeller shat bolts | T | T | T | T | T | T | T | T | T | T | T |
| Automatic transmission fluid | n/a | I | n/a | n/a | R | n/a | n/a | I | n/a | n/a | R |
| Fuel pipes and hoses | I | I | I | I | I | I | I | I | I | I | I |
| BENZIN activated charcoal filter | n/a | I | n/a | n/a | I | n/a | n/a | I | n/a | n/a | I |
| Exhaust pipes and mountings | I | I | I | I | I | I | I | I | I | I | I |
| Transfer gear oil | n/a | I | n/a | n/a | R | n/a | n/a | I | n/a | n/a | R |
| Differential oil front axle | n/a | I | n/a | n/a | R | n/a | n/a | I | n/a | n/a | R |
| Differential oil rear axle | n/a | I | n/a | n/a | R | n/a | n/a | I | n/a | n/a | R |
| Engine mountings | n/a | I | n/a | n/a | R | n/a | n/a | I | n/a | n/a | R |
| Bolts and nuts on chassis and body | n/a | I | n/a | n/a | I | n/a | n/a | I | n/a | n/a | T |
| Visual check underbody for damages | I | I | I | I | I | I | I | I | I | I | I |
| Electrical System / Interior |  |  |  |  |  |  |  |  |  |  |  |
| Mirrors and mirror control | I | I | I | I | I | I | I | I | I | I | I |
| Wiper and washer system | I | I | I | I | I | I | I | I | I | I | I |
| Lights front | I | I | I | I | I | I | I | I | I | I | I |
| Lights rear | I | I | I | I | I | I | I | I | I | I | I |
| Light Off system | I | I | I | I | I | I | I | I | I | I | I |
| Seat belts, seats. Seat adjuster | I | I | I | I | I | I | I | I | I | I | I |
| Intercom/ PA-system | I | I | I | I | I | I | I | I | I | I | I |
| A/C functionality | I | I | I | I | I | I | I | I | I | I | I |
| A/C fresh air filter | C | R | C | C | R | C | C | R | C | C | R |
| A/C service | n/a | I | n/a | n/a | R | n/a | n/a | R | n/a | n/a | R |
| Brake pedal/ hand brake lever | I | I | I | I | I | I | I | I | I | I | I |
| Fire extinguisher | I | I | I | I | I | I | I | I | I | I | I |
| Radio systems | I | I | I | I | I | I | I | I | I | I | I |
| Visual check interior | I | I | I | I | I | I | I | I | I | I | I |
| Winch functionality | I | I | I | I | I | I | I | I | I | I | I |
| Winch rope condition | I | I | I | I | I | I | I | I | I | I | I |
| Road Test |  |  |  |  |  |  |  |  |  |  |  |
| Road test | I | I | I | I | I | I | I | I | I | I | I |
| Diesel smoke | I | I | I | I | I | I | I | I | I | I | I |

| Section 2: Soft Skin Vehicles Maintenance Schedule |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I = Inspect, C = Cleaning, D = Drain, R = Replace, T = Tighten, L = Lubricate |  |  |  |  |  |  |  |  |  |
| Maintenance interval x 1000 km | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 |
| Maintenance Category | E | E | F | E | E | G | E | E | H |
| Engine Compartment |  |  |  |  |  |  |  |  |  |
| Engine oil incl. Filter and air filter element | R | R | R | R | R | R | R | R | R |
| Engine control unit with a diagnostic computer | I | I | I | I | I | I | I | I | I |
| Tightness of cooling, power and exhaust systems, as well as the condition of hoses, pipelines and their connections | I | I | I | I | I | I | I | I | I |
| Spark plug | n/a | n/a | R | n/a | n/a | R | n/a | n/a | R |
| Timing belt | n/a | n/a | n/a | n/a | n/a | R | n/a | n/a | n/a |
| Visual check of engine compartment | I | I | I | I | I | I | I | I | I |
| Body |  |  |  |  |  |  |  |  |  |
| Clutch drive | I | I | I | I | I | I | I | I | I |
| Condition of protective covers of hinges of drives of wheels | I | I | I | I | I | I | I | I | I |
| Tires condition /rotation/ and tire pressure | I | I | R | I | I | I | I | I | R |
| Condition of bushings and pillows of stabilizers roll stability, silent blocks suspension arms | I | I | I | I | I | I | I | I | I |
| Condition of the ball joints | I | I | I | I | I | I | I | I | I |
| Condition of the front and rear shock absorbers | I | I | I | I | I | I | I | I | I |
| Level in the power steering reservoir | I | I | I | I | I | I | I | I | I |
| Adjustment of all doors/ readjust | I | I | I | I | I | I | I | I | I |
| Tightness of system of the hydraulic booster of a steering | T | T | T | T | T | T | T | T | T |
| Condition of covers of the steering mechanism and tips of steering draughts | I | I | I | I | I | I | I | I | I |
| Window lifter/ hydraulic tailgate | I | I | I | I | I | I | I | I | I |
| Interbody |  |  |  |  |  |  |  |  |  |
| Brake fluid level in the reservoir | I | I | I | I | I | I | I | I | I |
| Brake pads and discs | I | I | I | I | I | I | I | I | I |
| Hydraulic brakes, condition of tubes and hoses | I | I | I | I | I | I | I | I | I |
| Condition of the pads and discs of the brake mechanisms of the front wheels | I | I | I | I | I | I | I | I | I |
| Condition of the windshield and rear-view mirrors | I | I | I | I | I | I | I | I | I |
| Condition of brushes of cleaners of a windshield and glass of a door of a luggage compartment | I | I | I | I | I | I | I | I | I |
| Anti-corrosion coating | I | I | I | I | I | I | I | I | I |
| Test and lubrication of the hood latch | L | L | L | L | L | L | L | L | L |
| Visual check underbody for damages | I | I | I | I | I | I | I | I | I |
| Electrical System / Interior |  |  |  |  |  |  |  |  |  |
| Checking the battery with a tester | I | I | I | I | I | I | I | I | I |
| Outdoor and indoor lighting lamps | I | I | I | I | I | I | I | I | I |
| Signalling devices in the instrument cluster | I | I | I | I | I | I | I | I | I |
| Ventilation, Heating and Air Conditioning System |  |  |  |  |  |  |  |  |  |
| A/C functionality | I | I | I | I | I | I | I | I | I |
| A/C fresh air filter | R | R | R | C | C | R | C | C | R |
| Refrigerant | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | R |