# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply and Delivery of 100 handheld Global Positioning System (GPS) devices

Publication reference: EUBAM-26-15

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | GLOBAL POSITIONING SYSTEM/GPS Minimum Technical Requirements 1/PHYSICAL CHARACTERISTICS: - Dimensions: 6.1 x 16.0 x 3.6 cm +/- 5% - Display: transflective; 65k color - Display resolution: 160 x 240 pixels - Display size: 3.6 x 5.5 cm +/- 5% - Weight: less than 250g - Battery: - Type: AA - Life: minimum 12 hours - Receiver: highly sensitive - Memory/history: 16 GB at minimum - Interface: - High speed mini-USB - NMEA 0183 compatible 2/MAPS and MEMORY: - Ability to add maps - Automatic routing - MAP segments: minimum 2500 - Tracks: minimum 150 - Waypoints: minimum 4500 - External memory storage: micro-SD card 3/SENSORS: - GPS + GLONASS + GALILEO - Barometric altimeter - Compass: tilt compensated 3 -axis 4/OUTDOOR CAPACITIES: - Point to point navigation - Area calculation - Sun and moon information - Picture viewer 5/CONNECTIVITY: - Wireless connectivity: - Bluetooth - ANT+ 6/DOCUMENTATION: - User manual provided with each delivered GPS. - English language 7/WARRANTY: - Minimum factory warranty of 1 year All equipment shall be commercially available, non-lethal and intended for civilian operational use in border monitoring and inspection activities. Equivalent or superior specifications meeting the functional requirements shall be accepted. INCOTERM: DDP EUBAM LIBYA PALM CITY/TRIPOLI LIBYA QTY: 100 (ONE hundred) |  |  |  |