# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply and Delivery of 30 Civilian Drones

Publication reference: EUBAM-26-17

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | CIVILIAN DRONES 1/AIRCRAFT: - Type : quadrirotor - Flight time : 25 mn minimum up to 45 mn - Flight distance : up to 20 km - Global navigation satellite system: GPS+ Galileo minimum - Internal storage : 2 GB at minimum 2/CAMERA: - Type : day and night - Image sensor: CMOS - Photo format: JPEG/DNG/RAW - Video resolution: 4 K - Video format: MP4 - Digital zoom: - 12MP Photo : 1-3 x - 4K : 1-3 x - FHD: 1-4 X - Transmission distance : - Urban aera: 1- 4 km - Suburban aera : 4 - 10 km - Seaside : 10 -20 km 3/GIMBAL: - 3 axis – tilt/roll/pan at minimum 4/SENSING: - Omnidirectional vision system with 3D infrared sensor 5/VIDEO TRANSMISSION: - Video transmission system : Digital HD - Live view qulity :HD - Remote controller 6/BATTERY: - Type, Li-ion - Charger 7/STORAGE: - One (01) Micro SD card 256GB 8/DOCUMENTATION: - User manual provided with each delivered drone. - Language: English mandatory; Arabic optional 9/WARRANTY: - Minimum factory warranty of 1 year INCOTERM: DDP EUBAM LIBYA PALM CITY/TRIPOLI LIBYA QTY: 30 (Thirty) |  |  |  |